	Stainless Steel Wire Mesh

Materials: SUS302, 304,304L, 316, 316L.
Weaving : plain weave, twill weave.
Character: acid-resisting , alkali-resisting, heat-resisting, wear-resisting etc.
Uses: Used in mine, oil, chemical industry, food industry, pharmaceutical industry and machine producing etc.
Our factory can produce all kinds of products according to the customers' requirements.


[image: image1.jpg]



 

 

Mesh

Wire Dia. 

Aperture

Inch

mm

Inch

mm

1x1 

.080 

2.03 

.920 

23.37 

2X2 

.063 

1.60 

.437 

11.10 

3X3 

.054 

1.37 

.279 

7.09 

4X4 

.063 

1.60 

.187 

4.75 

4X4 

.047 

1.19 

.203 

5.16 

5X5 

.041 

1.04 

.159 

4.04 

6X6 

.035 

.89 

.132 

3.35 

8X8 

.028 

.71 

.097 

2.46 

10X10 

.025 

.64 

.075 

1.91 

10X10 

.020 

.51 

.080 

2.03 

12X12 

.023 

.584 

.060 

1.52 

12X12 

.020 

.508 

.063 

1.60 

14X14 

.023 

.584 

.048 

1.22 

14X14 

.020 

.508 

.051 

1.30 

16X16 

.018 

.457 

.0445 

1.13 

18X18 

.017 

.432 

.0386 

.98 

20X20 

.020 

.508 

.0300 

.76 

20X20 

.016 

.406 

.0340 

.86 

24X24 

.014 

.356 

.0277 

.70 

30X30 

.013 

.330 

.0203 

.52 

30X30 

.012 

.305 

.0213 

.54 

30X30 

.009 

.229 

.0243 

.62 

35X35 

.011 

.279 

.0176 

.45 

40X40 

.010 

.254 

.0150 

.38 

50X50 

.009 

.229 

.0110 

.28 

50X50 

.008 

.203 

.0120 

.31 

60X60 

.0075 

.191 

.0092 

.23 

60X60 

.007 

.178 

.0097 

.25 

70X70 

.0065 

.165 

.0078 

.20 

80X80 

.0065 

.165 

.0060 

.15 

80X80 

.0055 

.140 

.0070 

.18 

90X90 

.005 

.127 

.0061 

.16 

100X100 

.0045 

.114 

.0055 

.14 

100X100 

.004 

.102 

.0060 

.15 

100X100 

.0035 

.089 

.0065 

.17 

110X110 

.0040 

.1016 

.0051 

.1295 

120X120 

.0037 

.0940 

.0064 

.1168 

150X150 

.0026 

.0660 

.0041 

.1041 

160X160 

.0025 

.0635 

.0038 

.0965 

180X180 

.0023 

.0584 

.0033 

.0838 

200X200 

.0021 

.0533 

.0029 

.0737 

250X250 

.0016 

.0406 

.0024 

.0610 

270X270 

.0016 

.0406 

.0021 

.0533 

300X300 

.0051 

.0381 

.0018 

.0457 

325X325 

.0014 

.0356 

.0017 

.0432 

400X400 

.0010 

.0254 

.0015 

.370 

500X500 

.0010 

.0254 

.0010 

.0254 

635X635 

.0008 

.0203 

.0008 

.0203 


	


